EXHIBIT 2-V

HUD LEAD-BASED PAINT REQUIREMENTS FOR

HOUSING REHABILITATION PROJECTS

HUD’s LBP requirements do not apply if any one of the following exist: - The structure was built after January 1, 1978; or - Is officially certified as free of lead-based paint by an EPA-certified lead-based paint inspector; or - A visual inspection identifies no deteriorated paint, and rehabilitation work shall disturb - less than two square feet (2 sq. ft.) of paint in any one interior room; - less than 10% of the total surface area on any interior component; and - less than 20 sq. ft. on exterior painted surfaces

| # Amount of # CDBG Funds | #### Lead-Based Paint Requirements |
| --- | --- |
| - Property receiving $5,000 or less in rehabilitation work per unit | - Notification: - Grantees must supply occupants with the lead information pamphlet (Protect Your Family From Lead In Your Home) and document its delivery. See Exhibit 2-X, CDBG Manual. - Grantees must notify owners of defective paint surfaces. - Owners must disclose the presence of LBP to current and prospective homebuyers/occupants. See Exhibit 2-Y.3, CDBG Manual. - Paint Testing: An EPA-certified LBP Inspector or Assessor must inspect the unit(s). Grantees may presume the presence of lead-based paint in lieu of conducting an inspection. i (See Notes, page I-4 below.) - Lead Hazard Reduction/Control: - HUD-qualified workers must perform the stabilization work (implementation of interim controls). These persons do not need to be EPA-certified LBP supervisors or workers. ii (See Notes, page I-4 below.) - Use safe work practices during rehabilitation. - Clearance: (Necessary only for the LBP hazard reduction work area.) An EPA-certified LBP Risk Assessor or Clearance Technician must certify the unit as safe for occupancy. i - Notification: Owners, occupants, and prospective occupants or purchasers must be notified of the results of any lead hazard reduction work. See Exhibit 2-Y.4, CDBG Manual. |

| - Property receiving more than $25,000 per unit in rehabilitation work | - Notification: - Grantees must supply occupants with the lead information pamphlet (Protect Your Family From Lead In Your Home) and document its delivery. See Exhibit 2-X, CDBG Manual. - Grantees must notify owners of defective paint surfaces. - Owners must disclose the presence of LBP to current and prospective homebuyers/occupants. See Exhibit 2-Y.3, CDBG Manual. - Paint Testing: An EPA-certified LBP Inspector or Assessor must inspect the unit(s). Grantees may presume the presence of lead-based paint in lieu of conducting an inspection. i 3) Risk Assessment: A Risk Assessment by an EPA-certified LBP Risk Assessor is required. i 4) Abatement (Removal) of LBP hazards: - Rehabilitation will include mandatory abatement (removal) of all LBP. - EPA-certified persons must do all work. EPA-certified Inspectors, Risk Assessors, Supervisors and Workers are the only persons who may do work on a project involving greater than $25,000 worth of rehabilitation on a unit. i Note: A HUD-qualified worker may do the abatement work, but an EPA-certified supervisor must be on the work site at all times. ii 5) Clearance: The LBP abatement work area must be cleared to assure it is safe for occupancy. An EPA-certified Risk Assessor or Clearance Technician must do the clearance. i 6) Notification: Owners, occupants and prospective occupants or purchasers must be notified of the results of any lead hazard reduction work. See Exhibit 2-Y.4, CDBG Manual. 7) Ongoing LBP maintenance: The property owners are responsible for maintaining a lead-free environment. If, upon annual inspection (if such an inspection is required by the property’s on-going funding sources), the presence of chipping, peeling or cracking paint is identified, EPA-certified personnel must inspect and assess the lead risk in the unit; HUD-certified workers must stabilize the deteriorated paint; and an EPA-certified person must perform clearance testing. |
| --- | --- |

i To be considered an EPA-certified inspector, assessor, clearance technician or supervisor, one must complete a weeklong training course and successfully pass the EPA-administered exam.

ii HUD-certified workers must complete an eight-hour, HUD-sponsored lead safety-training program. Contact your HOME Program Officer for current information on HUD training, or go to: http://www.leadsafetraining.org/

CDBG funds may not provide assistance to a home with a lead-based paint hazard, unless the hazard is controlled through stabilization, interim controls, or abatement (removal).

Choose either to:

Presume LBP is present, or

Conduct a LBP inspection

Has the property had a lead-based paint inspection by an EPA-certified Inspector or Assessor, and found to be free of lead-based paint?

Conduct visual assessment. Notify homeowner of any deteriorated paint, provide LBP safety pamphlet.

Does the visual assessment identify deteriorated paint?

Will rehabilitation activities disturb more than:

two square feet (2 sq. ft.) of paint in any one interior room;

10% of the total surface area on any interior component; or

20 sq. ft. on exterior painted surfaces?

The project does not contain lead-based paint. Proceed with rehabilitation activities.

The project will not disturb lead-based paint. Proceed with rehabilitation activities.

NO

NO

NO

NO

YES

or

YES

YES

YES

YES

Lead-based Paint Flowchart

for Single- and Multi-Family Rehabilitation

START

Was the property constructed in 1978 or later?

Conduct inspection by EPA-certified Inspector or Assessor

Presume lead-based paint is present

Is the unit found free of lead-based paint?

NO

The lead hazard requires stabilization, interim controls, or abatement, and must pass a clearance test following work. See EXHIBIT 2-V for specific requirements.

The lead hazard requires stabilization, interim controls, or abatement, and must pass a clearance test following work. See Appendix I, pages 1-4 for specific requirements.

CDBG funds may not provide assistance to a home with a lead-based paint hazard, unless the hazard is controlled through stabilization, interim controls, or abatement (removal).  See Appendix I, pages 1-4 for specific requirements.

START

Was the property constructed in 1978 or later?

The project does not contain lead-based paint. Proceed with rehabilitation activities.

NO

NO

NO

YES

YES

YES

Lead-based Paint Flowchart for Homebuyer

Has the property undergone a lead-based paint inspection and found to be free of lead-based paint?

Conduct visual assessment. Notify homeowner of any deteriorated paint, provide LBP safety pamphlet.

Does the visual assessment identify deteriorated paint?

Document that deteriorated paint was not found in interior or on exterior of house, and place statement in client file. Must include:

Date of visual inspection

Inspector signature

Date inspector completed the on-line visual assessment training course at: http://www.hud.gov/offices/lead/training/index.cfm

Assistance